User Guide

Menu

一、Element description…………………3

二、Call Library Function Node…………6

三、Operating procedures………………10

四、Q&A………………………………11

Element description

## Single Capture

Capture samples one time using the current Acquisition Settings and Trigger Conditions.

## Stop

Stop an ongoing acquisition. Show the newly acquired data

## Sample Rate

The sample rate or acquisition frequency determines how often samples are taken.

## Sample Depth

Determine the amount of data to be acquired per channel.

## Trigger Position

The trigger position determines which samples are stored. At the default 10%, 10% of the available memory is allocated to pre-trigger data and 90% to post-trigger data.

## Scan State

1.Ready to scan

2. scanning

3.Scan finish and show the signal

End

End program

Go to page

Read LA capture data

Page

Set read data range

Page size

Set read data size

Trigger setup message

If trigger setup has error setting, it will show the error message.

Device check message

If analyzer connect, it will show device ready.

## Trigger Setup

Trigger Setup offers settings trigger condition.

## Waveform

In Waveform view, the state of each channel is shown as a trace that changes between high and low depending on the state of the signal. This is the default view mode.

Call Library Function Node

New Call Library Function Node

Mouse right click -> functions -> Connectivity->Libraries & Executables -> Call Library Function Node

Function 1-input capture condition
Function name:

GetWaveData64Channel

Function 3-Read capture data
Function name:

StopAnalyzer

Function 2-Check capture finish every second
Function name:

CheckDeviceBusy

All functions are load ZpLvMfcDllApi.dll

Function 1-input capture

Input sample rate, sample depth, trigger setup, trigger position.

Function 2- Check capture finish every second

If capture finish return 1，else return 0

Function 3- Read capture data

Read capture data each 32 channel.

Operating procedures

1.Run labview

2. Set capture condition(sample rate, sample depth, trigger setup, trigger position)

3. Click Start button start capture.

4. Auto stop or click stop button to finish capture.

Run labview

Set capture condition

Click Start button

Finish capture

or click stop

Auto stop

4

1

3

2